# Handoff For Next Session

## Current Project Location

Main project folder:

/home/enders/Documents/PlatformIO/Projects/fsr9

Important files:

src/main.cpp

platformio.ini

web-ui/index.html

web-ui/bridge.py

host/so101_sensing/parser.py

tools/record_sensor_log.py

ros2_ws/src/so101_sensing_bridge

docs/software_architecture.md

## Current System State

The project now has a first structured software stack for the SO-101 gripper sensing setup.

The ESP32-S3 firmware reads the 9-zone FSR array and streams structured serial `FRAME` messages. Calibration is saved in flash. The browser UI can visualize live pressure data. Python host tools can parse and record data. A ROS 2 bridge scaffold exists.

The IMU fields are already included in the serial frame, but the real IMU driver is not implemented yet because the exact IMU module is not known.

## Verified Commands

Firmware build:

cd /home/enders/Documents/PlatformIO/Projects/fsr9

/home/enders/.platformio/penv/bin/pio run

Parser tests:

cd /home/enders/Documents/PlatformIO/Projects/fsr9

python3 -m unittest discover -s test -p 'test_*.py'

Both passed during the previous session.

## Useful Run Commands

Upload firmware:

cd /home/enders/Documents/PlatformIO/Projects/fsr9

./scripts/upload_safe.sh /dev/ttyUSB0

Start browser UI:

cd /home/enders/Documents/PlatformIO/Projects/fsr9

./scripts/start_ui.sh /dev/ttyUSB0

Open:

http://127.0.0.1:8090

Record JSONL:

cd /home/enders/Documents/PlatformIO/Projects/fsr9

python3 tools/record_sensor_log.py --port /dev/ttyUSB0 --format jsonl

Record CSV:

cd /home/enders/Documents/PlatformIO/Projects/fsr9

python3 tools/record_sensor_log.py --port /dev/ttyUSB0 --format csv

## Next Questions To Ask User

Ask these before implementing the next hardware-specific stage:

- 1. What exact IMU module is being used?

- 2. Is the IMU connected by I2C or SPI?

- 3. Which ESP32-S3 pins are used for the IMU?

- 4. How are the two IMX335 cameras connected to the host computer?

- 5. What OS and computer will run the camera and ROS stack?

- 6. Is ROS 2 installed? If yes, which distro?

- 7. Is LeRobot already installed for SO-101?

## Recommended Next Implementation Step

The best next implementation step is to add the real IMU driver.

Work plan:

- 1. Confirm IMU model and wiring.

- 2. Add the required PlatformIO library to `platformio.ini`.

- 3. Initialize the IMU in `setup()`.

- 4. Replace the `readImuSample()` stub in `src/main.cpp`.

- 5. Verify serial `FRAME` lines show `imu_status=1`.

- 6. Update parser tests with real IMU field expectations if needed.

- 7. Record a short JSONL log with FSR plus IMU data.

## After IMU Works

The next major step should be synchronized camera and robot data recording.

Suggested architecture:

- ESP32-S3 publishes FSR plus IMU serial frames.

- Host Python process reads ESP32 serial data.

- Host captures both IMX335 camera streams.

- SO-101 software logs joint state and gripper action.

- All data is timestamped and saved into one experiment folder.

Recommended experiment folder format:

data/raw/episode_YYYYMMDD_HHMMSS/

sensor.jsonl

camera_gripper/

camera_table/

robot_actions.jsonl

metadata.json

## Important Notes

- Do not route the IMX335 cameras through the ESP32-S3. They should connect to the host computer.

- The IMU chip temperature usually measures board/chip temperature, not true object temperature.

- If true object temperature is required, add a contact thermistor, digital temperature sensor, or IR temperature sensor near the gripper pad.

- The FSR array provides a contact-pressure map, not a full object shape. Shape estimation should combine FSR data with camera images and robot pose.

## Good First Goal Next Time

Get one clean recorded episode:

- 1. ESP32-S3 streaming valid FSR frames.

- 2. IMU fields active.

- 3. Browser UI showing pressure.

- 4. Recorder saving JSONL.

- 5. Manual object grasp performed.

- 6. Log file reviewed for pressure, centroid, and vibration changes.

That episode becomes the reference dataset for the next stage.